HA NOI UNIVERSITY OF SCIENCE AND TECHNOLOGY SCHOOL OF ELECTRICAL AND ELECTRONIC ENGINEERING PROJECT ELECTRONIC DESIGN II TOPIC: MIXER DESIGN FOR GSM-900 Instructor: DR. NGUYEN NAM PHONG Students: NGUYEN LE TRUNG 20186076 VU HOANG LONG 20182926 PHAM HUY HOANG 20182544 DANG ANH TU 20182938 NGUYEN VO PHUONG THAO 20182936 Ha Noi, 12-2021

PREFACE

Tài liệu này được Viện Điện tử - Viễn thông, trường Đại học Bách Khoa Hà Nội soạn thảo và ban hành nhằm mục đích hướng dẫn sinh viên trình bày đồ án tốt nghiệp một cách khoa học và thống nhất. Bản thân tài liệu này được biên soạn phù hợp với các quy định về trình bày một đồ án tốt nghiệp. Vì vậy, sinh viên có thể sử dụng trực tiếp mẫu này như một template khi viết quyển đồ án của mình. Sinh viên cũng được khuyến khích tham khảo hoặc sử dụng mẫu này khi viết báo cáo thực tập, báo cáo kết quả nghiên cứu, và các đồ án môn học khác.

Chi tiết về nội dung của phần Lời nói đầu được trình bày trong Mục 1.3.3.

TABLE OF CONTENTS

ABBREVIATIONS	i

FIGURES AND DIAGRAMS	ii

TABLES	iv

ABSTRACT	v

CHAPTER 1. INTRODUCTION	1

1.1 What is a mixer	1

1.2 Operating Principle	3

1.3 What is GSM 900?	5

1.4 Important Parameters of a Mixer.	6

1.4.1 Conversion loss	6

1.4.2 Noise Figure	7

1.4.3 High-side and Low-side Injection:	7

1.4.4 Third Order Intercept Point (IP3 or TOI)	7

1.4.5 1-dB Compression Point	8

1.4.6 Isolation	9

1.4.7 Linearity	9

1.4.8 Image Frequency	9

CHAPTER 2. DC – AC ANALYSIS	10

CHAPTER 3. S-PARAMETER SIMULATIONS	19

CHAPTER 4. MATCHING	19

CHAPTER 5. MIXER CONVERSION GAIN SIMULATION	20

5.1 Mixer conversion gain versus LO drive level	20

5.2 Mixer conversion gain versus RF signal level	23

CHAPTER 6. THIRD-ORDER INTERCEPT POINT AND NOISE FIGURE ………….25

6.1 Third-order Intercept Point	25

6.2 Noise Figure	28

CONCLUSION	29

Kết luận chung	29

Hướng phát triển	29

Kiến nghị và đề xuất	29

CONTRIBUTION	30

REFERENCES	31

APPENDIX	33

Phụ lục 1. Mẫu trang bìa chính của đồ án	33

# ABBREVIATIONS

| AC BJT DC FDD FDMA FET GSM HB IF LO LPF NF RF SNR TDMA TOI/IP3 | Alternating Current Bipolar Junction Transistor Direct Current Frequency Division Duplex Frequency Division Multiple Access Field-Effect Transistor Global System for Mobile Communications Harmonic Balance Intermediate Frequency Local Oscillator Low Pass Filter Noise Figure Radio Frequency Signal-to-Noise Ratio Time Division Multiple Access Third Order Intercept Point |
| --- | --- |

# FIGURES AND DIAGRAMS

Figure 1-1: Mixer	2

Figure 1-2: (a) Downconversion by mixing, (b) resulting spectra, (c) use of LNA to reduce noise	3

Figure 1-3: (a) Constant-LO and (b) constant-IF downconversion mixing	4

Figure 1-4: Problem of image in heterodyne downconversion.	5

Figure 1-5: GSM900 System	6

Figure 1-6: High-side and Low-side Injection	7

Figure 1-7: Third order Intercept Point	8

Figure 1-8: 2-Tone Downconversion Spectrum	8

Figure 1-9: 1-dB Compression Point	9

Figure 1-10: Image Frequency	9

Figure 2-1 single ended active mixer example	10

Figure 2-2 DC curve verification simulation	11

Figure 2-3: Comparison of Measured (blue) and Simulated (red) DC I-V Curve	12

Figure 2-4 bias network schematic	13

Figure 2-5: schematic of DC bias network	15

Figure 2-6: simulation result of DC bias network	16

Figure 2-7: schematic gain compression	17

Figure 2-8: result of gain compression	18

Figure 5-1: Measure equation for P_IF	20

Figure 5-2: LO Drive Level and RF/IF Matching	21

Figure 5-3: Conversion Gain vs LO Power	22

Figure 5-4: ICE vs LO Power	23

Figure 5-5 Compression Point Simulation Configuration	23

Figure 5-6: Gain vs RF Power	24

Figure 5-7: 1-dB Compression Point	24

Figure 6-1: TOI simulation parameter	25

Figure 6-2: Harmonic Balance Configuration for TOI simulation	26

Figure 6-3: TOI Equations	26

Figure 6-4: Vout Spectrum	27

Figure 6-5: Vout spectrum (zoom in)	27

Figure 6-6: NF Simulation Configuration	28

Figure 6-7: NF vs LO Power	28

# TABLES

Table 2-1: VBE vs IBB, given VCC = 1V	13

Table 2-2: Calculated Rb and Rc	14

Table 6-1 Two-tone IP3 values at the output	26

# ABSTRACT

Tóm tắt đồ án, có độ dài 1-2 trang, được trình bày tại đây. Chi tiết xem Mục 1.3.9

# INTRODUCTION

## What is a mixer

- One of the most useful radio frequency processes is mixing, which is performed by a RF mixer. Unlike a conventional audio mixer where sounds are simply added together to create an interesting sound effects, RF mixing refers to a different process. Where signals are multiplied together to create a new signal with a different frequency.

- RF mixers are used in virtually every cellphones  and many other devices like radios, spectrum analyzers. It can change signal’s frequencies for different purposes. For example, with mixers a signal can be undertaken on a lower frequency where it is easier to perform signal processing. On contrary, the can also be increased to a higher frequency for transmitting.

- In electronic, a mixer, or frequency mixer, is a nonlinear electrical circuit that creates new frequencies from two signals applied to it. In its most common application, two signals are applied to a mixer, and it produces new signals at the sum and difference of the original frequencies. Other frequency components may also be produced in a practical frequency mixer.

- Mixers are widely used to shift signals from one frequency range to another, a process known as heterodyning, for convenience in transmission or further signal processing. For example, a key component of a superheterodyne receiver is a mixer used to move received signals to a common intermediate frequency. Frequency mixers are also used to modulate a carrier signal in radio transmitters.

- As the heart of the superheterodyne transceivers, mixer play an important role of selecting channels, where changing the local oscillator frequency would result in selecting different radio channel but still maintain the same intermediate frequency. Thus it can be used with same filters and signal processing method for different channels or frequencies.

Figure 1-1: Mixer

- A mixer contains 3 ports: Input signal, local oscillator and output signal:

If the output signal has frequency lower than the input signal’s, then the mixer will perform a down conversion transform, else it will perform an up conversion transform.

- There are 2 main type of mixer:

  - Passive mixers:   Passive mixers typically use passive components in the form of diodes as the switching element within the RF circuit. As a result they cannot exhibit any gain, but many forms can provide excellent levels of performance.

Passive mixers mainly use Schottky diodes because of their low turn-on voltage, but they require the use of a balun / RF transformer if they are to be used in a balanced or double balanced mixer. This can limit the frequency response.

  - Active mixers:   As the name of the Active RF mixer contains active electronic components like a bipolar transistor, FET or even a vacuum tube / thermionic valve. These types of RF mixer are able to provide gain as well as proving the multiplication or RF mixer capability.

- Mixers are also looked at by whether they are balanced or not. Balancing them requires baluns - balanced to unbalanced transformers - but this provides improvements in performance:

  - Unbalanced mixer

  - Single balanced mixer

  - Double balanced mixer

  - Triple balanced mixer

## Operating Principle

To lower the center frequency, the signal is multiplied by a sinusoid A0cos ωLOt, which is generated by a local oscillator (LO). Since multiplication in the time domain corresponds to convolution in the frequency domain, we observe from Fig. 4.7(b) that the impulses at ±ωLO shift the desired channel to ±(ωin ± ωLO). The components at ±(ωin ± ωLO) are not of interest and are removed by the low-pass filter (LPF) in Fig. 4.7(a), leaving the signal at a center frequency of ωin - ωLO. This operation is called “downconversion mixing” or simply “downconversion.” Due to its high noise, the downconversion mixer is preceded by a low-noise amplifier [Fig. 4.7(c)].

Figure 1-2: (a) Downconversion by mixing, (b) resulting spectra, (c) use of LNA to reduce noise

Called the intermediate frequency (IF), the center of the downconverted channel, ωin - ωLO, plays a critical role in the performance. “Heterodyne” receivers employ an LO frequency unequal to ωin and hence a nonzero IF.

How does a heterodyne receiver cover a given frequency band? For an N-channel band, we can envision two possibilities. The LO frequency is constant and each RF channel is downconverted to a different IF channel [Fig. 4.8(a)], i.e., fIFj =fRFj - fLO. The LO frequency is variable so that all RF channels within the band of interest are translated to a single value of IF [Fig. 4.8(b)], i.e., fLOj =fRFj - fIF. The latter case is more common as it simplifies the design of the IF path; e.g., it does not require a filter with a variable center frequency to select the IF channel of interest and reject the others. However, this approach demands a feedback loop that precisely defines the LO frequency steps, i.e., a “frequency synthesizer”.

Figure 1-3: (a) Constant-LO and (b) constant-IF downconversion mixing

Problem of Image Heterodyne receivers suffer from an effect called the “image.” To understand this phenomenon, let us assume a sinusoidal input and express the IF component as

AcosωIFt=Acosωin-ωLOt  =AcosωLO-ωint

That is, whether ωin-ωLO is positive or negative, it yields the same intermediate frequency. Thus, whether ωin lies above ωLO or below ωLO, it is translated to the same IF. Figure 4.9 depicts a more general case, revealing that two spectra located symmetrically around ωLO are downconverted to the IF. Due to this symmetry, the component at ωim is called the image of the desired signal. Note that ωim= ωin + 2ωIF = 2ωLO - ωin.

Figure 1-4: Problem of image in heterodyne downconversion.

What creates the image? The numerous users in all standards (from police to WLAN bands) that transmit signals produce many interferers. If one interferer happens to fall at ωim = 2ωLO - ωin, then it corrupts the desired signal after downconversion.

While each wireless standard imposes constraints upon the emissions by its own users, it may have no control over the signals in other bands. The image power can therefore be much higher than that of the desired signal, requiring proper “image rejection.”

## What is GSM 900?

- GSM900 is a version of Global System for Mobile Communications, a popular second generation cellular system, which offer full-duplex voice communication service and operate at  900 MHz band

- The 900 MHz band defined in the ETSI standard includes the primary GSM band (GSM-P), the extension (see E-GSM) and the part of the 900 MHz band that is reserved for railways (R-GSM).

- The total GSM900 band defined in the standard ranges from 876 - 915 MHz paired with 921 - 960 MHz. Mobiles transmit in the lower band and base stations transmit in the upper band.

- In daily life, the term GSM900 band is used for the parts of the band that are used by the GSM operators to offer public services, which exludes the R-GSM band. This part of the band that remains ranges from 880 - 915 MHz paired with 925 - 960 MHz band.

- The system adopts FDD scheme with duplexing distance of 45 MHz. The multiple access scheme is a combination of FDMA and TDMA, where the spectrum for each transmission irection is divided into frequency subband of 200 kHz and each of the subband carries 8 voice channels in a time division.

Figure 1-5: GSM900 System

## Important Parameters of a Mixer.

### Conversion loss

The conversion loss of a mixer is the ratio of the desired IF output (voltage or power) to the RF input signal value (voltage or power), corresponding to each level of the LO.

If the input impedance and the load impedance of the mixer are both equal to the source impedance, then the voltage conversion gain and the power conversion gain of the mixer will be the same in dB’s.

The Conversion Loss of the RF Mixer measured in dBm is given by:

- For Lower Sideband:

CLLSB= PDiffPRF=PDiff[dBm]- PRF[dBm]   PDiff is the power of fIF=fLO-fRF

- For Upper Sideband:

CLUSB= PSumPRF=PSumdBm- PRFdBm   PSum is the power of fIF=fLO+fRF

- Generally:

ConversiondB=Output IF power delivered to the loaddBm-RF input power[dBm]

Figure 1-6: High-side and Low-side Injection

The typical conversion gain of an Active Mixer is approximately +10dB, when the conversion loss of a typical Diode Mixer is approximately -6dB.

### Noise Figure

Noise Figure is the ratio between the Signal to noise ratio (SNR) at the input and at the output.

NF= SNRinSNRout

### High-side and Low-side Injection:

- If FLO > FRF: High-side injection. The LO will be called as high-side LO (HSLO).

- If LO < FRF: Low-side injection. The LO will be called as low-side LO (LSLO).

### Third Order Intercept Point (IP3 or TOI)

IP3 is an important parameter to consider the linearity of the mixer. It is the fictional value defined by the extrapolated intersection of the primary IF response with the two-tone 3rd intermodulation IF product that results when two RF signals are applied to the RF port.

Figure 1-7: Third order Intercept Point

Denote the local oscillator’s frequency as FLO then the IP3 products are (2F1 ± F2) ± FLO and (2F2 ± F1)± FLO. For the downconversion case, the significant noise products are (2F1 - F2) - FLO and (2F2 - F1) - FLO since they are near around the Intermediate Frequency (IF).

Figure 1-8: 2-Tone Downconversion Spectrum

### 1-dB Compression Point

Within the small input signal level, the output power increases linearly with the input one. If the input power continuously increases, the conversion loss will start to increase. The 1-dB Compression Point is where it gains 1-dB compared to the linear one. Mixer must be designed under this point to avoid the unexpected outputs (such as third or higher order products).

Figure 1-9: 1-dB Compression Point

### Isolation

Isolation is the amount of LO  power that leaks into either the IF or the RF ports, usually in dB. This is one of the most significant drawback when designing a transceiver system.

There are multiple types of isolation: LO-to-RF, LO-to-IF and RF-to-IF isolation.

### Linearity

The linearity of a mixer is the ability of controlling the signal level. The higher linearity a mixer gains, the upper its IP3 is.

### Image Frequency

Figure 1-10: Image Frequency

As mentioned above, fRF= fLO±fIF, hence there are several options for fRF. As the above figure, if we choose fRF > fLO (LSLO case), then aside, fIF we also obtain another unexpected signal at frequency fLO – fIF and so on. Those frequencies are called Image Frequency and we have to choose an appropriate filter to eliminate them out.

# DC – AC ANALYSIS

As mention in the previous part, GSM900 is the technology that usually be used on mobile devices, especially, cellphones, which battery lifetime is critical. Therefore, the mixer also needs to be power efficient, comparing between 2 kind of mixers, the active mixer having huge advantage over power consumption, which makes it suitable for implementing in the GSM900 devices.

2.1. Single-ended active mixer

The single ended mixer is one of the simplest active mixers. The mixing performed by  the mixing is performed by modulating the transconductance of the driver stage with the LO signals. Because of architecture simplicity, single ended mixers have lowest noise figure within active mixers

Figure 2-1 single ended active mixer example

Some parameters would be considered in mixer design (mentioned in previous chapter):

- Conversion loss

- Noise figure

- Isolation

- Linearity

2.2. Design a single-ended mixer

2.2.1. Selecting devices

The purposed design consists of a transistor, resistors, capacitors. For the transistor, the purposed candidate is the MMBR941 bipolar junction transistor from Motorola. It doesn’t have as good mixing properties as RF field-effect transistor, it’s very low cost and has accurate simulation model available with a reasonable performance. This BJT model is available with Keysight Advance Design System, where it can be verified and also evaluated the design.

For resistors and capacitors, they are just generic passive devices in the simulation.

2.2.2. DC curve verification

In order to verify the device under DC parameters, the I-V Curve of the transistor is needed. So as to plotting the I-V Curve of the transistor, the sweeping parameters function of ADS is used. Therefore, the device can be tested how it would behave under different value range and DC configuration. The parameters are tested in this part are Collector – Emitter Voltage and Base current of the transistor

Simulation sweeping setup:

| - Parameter: VCE - Start value: 0V - Sweeping step: 0.1V - Stop value: 6V | - Parameter: IBB - Start value: 50uA - Sweeping step: 75uA - Stop value: 350uA |
| --- | --- |

Figure 2-2 DC curve verification simulation

Since the DC controller can only sweep one value, the Parameter sweep will be used for sweeping IBB. Both parameters are set as variable for sweeping controller. The current probe is for measuring the change of Collector current for reference in the I-V Curve plot.

The simulation result:

Figure 2-3: Comparison of Measured (blue) and Simulated (red) DC I-V Curve

There are not much different between simulated and measured result, the curve show that at the specified operating point of VCE = 1V, ICE < 0.6mA the device will operate at low current region.

2.2.3. DC bias network

This part will help to select device operation point calculating the base and collector resistors. Therefore, the base current needs to be determined, as the collector voltage and current are already specified.

In this setup VCC will be fixed at 1V and IBB will be swept:

- Parameter: IBB

  - Start value: 1uA

  - Sweeping step: 0.5uA

  - Stop value: 10uA

Figure 2-4 bias network schematic

To mimic the real setup, the bias tee components (the DC_Feed and DC_Block components) and 50 Ω terminations are added. The bias point current is lower than the specified final value, because the device will have to work with large LO signal, which cause a shift in the DC component of the collector current.

Table 2-1: VBE vs IBB, given VCC = 1V

The figure x+3 shows that IBB need to be 5uA to have ICC= 514.6uA with corresponding VBE of 707.9V which is well below the specification

Table 2-2: Calculated Rb and Rc

To calculate Rb and Rc, the collector-emitter voltage of 0.75 V, at the point of IBB = 5uA, the correspond Rb and Rc are 8424.296 Ω and 481.125 Ω.

The next step will confirm this bias operation using these standard values of Rb and Rc and then verify the S-parameters of the model against measured value.

2.2.4. DC bias network verification

In this part, the bias configuration obtained from 2.1.3 will be verified. The schematic incluse bias tee components (DC feeds and blocks) to make sure the performance of RF

This simulation will run with both values of the exact resistor values and nearest standard values (Rc = 470 Ω, Rb = 8.2 kΩ) to verify the RF performance

Figure 2-5: schematic of DC bias network

The S-parameters controller will run the frequency sweep to see the circuit’s frequency response and and displayed, together with measured.

Figure 2-6: simulation result of DC bias network

There is a great alignment between values obtained here verifies the small-signal RF performance. The device compression point will be simulated next to confirm large-signal operation.

2.2.4. Large-Signal Verification (gain compression)

The output compression at frequency of 900MHZ will be calculated in this step. The first method to calculate is using Harmonic Balance controller, by sweeping the input power level from low to high values until the output power compresses (the ratio Pout/Pin starts to fall off from its small-signal value. The input power variable, “PwrIn” is swept from -45 to -5 dBm and a Measurement Equation component is used to define the output power at 900 MHz, in dBm.

The dBm function assumes the power is being delivered to a 50 Ω load. The argument “HB.Vout[1]”, specifies the fundamental frequency.

The second method is more direct and does not require graphs or sweeping variables. The Gain Compression controller “XDB” performs a harmonic balance analysis that correctly calculates and outputs the input and output power levels at the specified compression point. the output from this method: the input and output power levels at 1dB compression are listed in dBm.

Figure 2-7: schematic gain compression

Figure 2-8: result of gain compression

The result graphs show both calculate method of the simulation agreed with the measured data.

# S-PARAMETER SIMULATIONS

We continue mixer testing by taking several S-parameter measurements and find network values that match.

3.1. S-parameter

S-parameters are complex numbers that can be used directly or in a matrix to display reflection/transmission characteristics in the frequency domain (amplitude and optionally phase).

It is terminated with the component or circuit's characteristic impedance, which is generally 50 ohms for most RF work.

The arrangement of S-parameters and the resulting scattering matrix measures how RF energy propagates through a multi-port network. It provides for accurate characterization and description of component and circuit propertises, including parasitic and stray values. This eliminates the need to model the circuit's impedances, resistors, capacitances, and inductances at higher frequencies, when such modeling would be extremely difficult and likely miss many real-world problems.

3.2. S-parameter simulation

3.2.1. Set up

Simulation S-parameter setup:

- Start point: 45 MHz

- Stop point: 900MHz

- Step value: 855MHz

To represent the impedance of the local osscilator (LO), adding series RC to ground:

R= 50 Ohms; C= 0.5pF and insert these components onto base of transistor.

Figure 3.1: LO components: series R-C to ground

3.2.2. Simulate S2,1

As mentioned before, S2,1 is the measure of signal flow coming out relative to the RF stimulus entering in.

In figure 3.3, the plot of S2,1 with the frequency range from 100MHz to 2 GHz (100 MHz steps) shows results before and after deactivating the LO components.

Clearly in this figure, the S2,1 decreases nearly 0.6dB in circuit with LO impedance.

(a)

(b)

Figure 3.2: Plot S2,1 (a) with LO impedance and (b) with deactivated LO components

3.2.3. Plot S1,1 impedance

The S-11 trace is depicted in a Smith chart

The marker displays the reflection coefficient with magnitude/phase as well as the impedance (real and imaginary).

Figure 3.3: S-11 Smith chart

Figure 3.4: List of four S-parameters

Rather than picking just one of the S values, we can use a tabular structure to see all four of S-parameters. In addition, the termination's port Z=50 Ohms impedance is provided.

3.2.4. Frequency sensitive termination for RF and IF

Frequency sensitive Z ports to specify how a termination reacts to the variety of frequency.

For S-11 matching, the Z port on the output is designed to be an RF short to ground, similar to a filter. The Z port at the input, on the other hand, is set to be an IF short to ground. After generating the matching networks, this offers a better estimate of the final S-parameters.

Figure 3.5: The Z port at the input

The Equation Based-Linear Z port is designed to be a short at the IF frequency, using when building the output matching network S-2,2

2.2.4. DC feed inductors and DC block capacitor

The DC path must be protected from RF signals, and lower frequency signals must be blocked from RF signals because feedback from the drain to the gate might produce oscillation. DC feed components behaves as ideal inductors short at DC and as an open circuit (impedance at any AC signal) at all other frequencies.

A DC feed inductor has an inductance value of L= ∞, whereas ordinary inductors have a limited L value.

Figure 3.6. DC componenst in design

Connect the transistor base to the resistor Rb with a DC feed inductor, then connect the collector to the resistor Rc with another DC feed inductor.

# MATCHING

# MIXER CONVERSION GAIN SIMULATION

In this chapter, we will try to simulate conversion gain of the mixer.

## Mixer conversion gain versus LO drive level

In this part, we will try to simulate conversion gain for the mixer and determine the effect of LO drive level on gain and DC bias.

The RF and LO frequencies and the LO power level have been defined as variables.

The RF drive level is specified at -50 dBm, while the harmonic balance controller is set up to sweep the LO drive level from -30 to -5 dBm. A simulation measurement equation defines the output power, in dBm, at the IF:

Figure 5-1: Measure equation for P_IF

This measurement equation defines it here instead of the data display page makes it possible to optimize for output IF power, if needed. The “mix” function will return the component of the Vout spectrum defined by {-1, 1}, meaning {-Freq[1] + Freq[2]} or -LO + RF = IF (45 MHz). The “P_IF” equation calculates the dBm value of the mix function.

Figure 5-2: LO Drive Level and RF/IF Matching

Since the conversion gain is the difference between P_IF and the RF, and the RF power is fixed at -50 dBm, the conversion gain can be calculated with a simple expression.

Note that the default dataset, LODrive, contains results for a 4.7 kΩ load resistor, and the conversion gain for this simulation is calculated by the equation “ConvGain”.

Figure 5-3: Conversion Gain vs LO Power

The conversion gain for a -10 dBm LO drive is 17 dB, which is unacceptably high. A second simulation was run with the load resistor reduced 1.5 kΩ, which creates a lossy mismatch on the output. The results for that simulation were output to dataset LOdrive15, and equation “ConvGain_Rl5kOhm” shows the conversion gain is reduced to 13.7 dB. This is still higher than the specification of 10 dB but will be left at this value for now since conversion gain can be expected to decrease further when non-ideal surface mount components replace the ideal components.

The graph below shown illustrates the effect of the LO drive level on DC bias. Increasing the LO signal at the base drives the output swing on the collector harder, shifting the DC component higher (see Figure 19). In practice, a 5 to 15 percent shift in collector bias current typically gives good performance for a mixer of this type.

Figure 5-4: ICE vs LO Power

## Mixer conversion gain versus RF signal level

The set-up for measuring mixer compression is very similar to LOdrive, except that the LO power level is now held constant at -10 dBm, while the RF power is swept from -50 to 0 dBm.

Figure 5-5 Compression Point Simulation Configuration

As the results in Figure below show, the mixer’s conversion gain reaches 1 dB compression at an input signal level of -27 dBm.

Figure 5-6: Gain vs RF Power

Figure 5-7: 1-dB Compression Point

# THIRD-ORDER INTERCEPT POINT               AND NOISE FIGURE SIMULATION

In this chapter, we will perform the simulations and related results of Third-order Intercept Point (TOI or IP3) and Noise Figure (NF).

## Third-order Intercept Point

In this section, we use the module Hamonic Balance Controller to figure out the value and characteristics of TOI based on our pre-fixed input power of LO and RF, also the chosen frequencies as mentioned in the INTRODUCTION chapter (fLO = 855 MHz, fRF = 900 MHz).

Initially, we define some required parameters by selecting out the module VAR. This VAR will initialize the values needed for the IP3 or TOI (third order intercept) simulation. The variable f_spacing is used to separate the 2 RF tones:

Figure 6-1: TOI simulation parameter

We also modify the source as above, note that we use the f_spacing / 2 to give 10 KHz offset spacing from the RF, so the actual spacing is 20 KHz.

For the Hamonic Balance Controller, we use Krylov solver to shorten the simulation time. We also allow the RF_pwr variable to be sent to the dataset.

Figure 6-2: Harmonic Balance Configuration for TOI simulation

Finally, we set up the IP3out measurement equation (TOI).

Figure 6-3: TOI Equations

Where hi_toi represents the higher spaced tone (RF_freq + spacing/2), and lo_toi is the remained one.

Running simulation, we obtain the following result:

Table 6-1 Two-tone IP3 values at the output

Trying to plot out the spectrum of Vout (denoted in the schematic), we will see the inter-modulation products (max order) generated by the two-tone simulation.

Figure 6-4: Vout Spectrum

It is hard to see the separation of m1 and m2, where m1 marks out lo_toi and m2 marks out hi_toi. Try to zoom in to see what happens:

Figure 6-5: Vout spectrum (zoom in)

## Noise Figure

The  Noise Figure (NF) measurement calculates the noise contributed by circuit for the given input frequency (RF) of the designed mixer. To perform it, we have to modify the HB controller and replace our last two-tone RF source with a single tone one.

Figure 6-6: NF Simulation Configuration

Running simulation, we obtain the following result:

Figure 6-7: NF vs LO Power

As you can see, The Noise Figure is acceptable as specification.

# CONCLUSION

## Kết luận chung

Xem Mục 1.3.12

## Hướng phát triển

(Nếu có)

## Kiến nghị và đề xuất

(Nếu có)

# CONTRIBUTION

- Theoretical Concepts and Plans: Team

- Simulation:

- DC – AC Analysis: 			Dang Anh Tu

- S – Parameters Analysis: 		Pham Huy Hoang

- Matching: 				Nguyen Vo Phuong Thao

- Conversion Gain: 			Nguyen Le Trung

- IP3 and NF: 				Vu Hoang Long

- Report and Proofreading: Team

# REFERENCES

- T. H. Cormen, C. E. Leiserson, and R. L. Rivet, Introduction to Algorithm. MIT Press, McGraw-Hill, 1990.

- J. W. DuBois, S. Schuetze-Coburn, S. Cumming, and D. Paolino, “Outline of discourse transcription,” in Talking Data: Transcription and Coding in Discourse Research, J. A. Edwards and M. D. Lampert, Ed. Hillsdale, NJ: Lawrence Erlbaum Associates, 1993, pp. 45-89.

- J. M. Airey, J. H. Rohfl, F. Brooks Jr., “Towards Image Realism with Interactive Update Rates in Complex Virtual Building Environments,” Comptuer Graphics, Vol. 24, No. 2, pp. 41-50, 1990.

- S. Brandt, G. Nutt, T. Berk, M. Humphrey, “Soft Real time Application Execution with Dynamic Quality of Service Assurance,” in Proceedings of the Sixth IEEE/IFIP International Workshop on Quality of Service, Hawaii, USA, May 1998, pp. 154-163.

- K. Riley, “Language theory: Applications versus practice,” presented at the Conf. of the Modern Language Association, Boston, MA, December 27-30, 1990.

- J. Jones. (1991). Networks (2nd ed.) [Online]. Available: http://www.atm.com.

References:

Understand RF Mixing & Frequency Mixers » Electronics Notes (electronics-notes.com)

Frequency mixer - Wikipedia

References:

1 - Giới thiệu mạng di động GSM (byethost7.com)

GSM900 - Telecom ABC

# APPENDIX

## Phụ lục 1. Mẫu trang bìa chính của đồ án

(Xem trang sau)

Cho thêm schematic, thông số bjt, v.v vào đây